## Six Sigma Black Belt

## Exam for 1.0 Define & 2.0 Measure with Answers

- Which of the following best describes statistical thinking:

  - All work is a series of interconnected processes.

  - All processes vary.

  - Understanding and reducing variation are keys to success.

  - All of the above.

- A functional deployment process map displays which of the following:  (select all that apply)

  - The cost of the current process,

  - The steps depicted in a process in sequential order,

  - The function in which each step is performed,

  - The name of the process owner.

- Quick wins should have the following characteristics: (select all that apply)

  - Easy, fast and cheap to Implement,

  - Has never been tried before,

  - Within the team’s control,

  - Approved by customer.

- Building ‘critical to quality’ (CTQ) measures begins with:

  - Sending a customer survey to all customers.

  - Developing a customer-focused business strategy.

  - Meeting with the Process Owner.

  - Selecting a Black Belt to work on the project.

- Examples of a CTQs are:

  - Cost/Unit, Productivity

  - Delivery time, purity, reliability.

  - Compliance with regulations and laws.

  - Safety, training hours.

- CTQs are often defined as ‘big Ys’ and ‘little ys’.  A characteristic of a ‘big Y’ is that it is big enough to:

  - Be of interest to customers.

  - Cost a lot of money.

  - Take a lot of time to improve.

  - Be seen at an operational business unit level.

- Effective teams have:

  - 4-6 team members, a black belt, a master black belt and a champion.

  - Sufficient budget and authority to operate as a team.

  - A charter, roles defined, team procedures, and a good working relationship.

  - Approvals by each leader, manager, stakeholder and champion.

- Why is it important that data be normally distributed?

  - So that the data will all look alike.

  - Because normality is an underlying assumption for applying many statistical tools.

  - Because normal data is easy to work with.

  - Because normal data are continuous.

- Hypothesis tests presume that there is a differences only if the p value is less than:

  - 0.1

  - 0.5

  - 0.05

  - 0.01

  - 0.95

- In qualitative process analysis, a process step is value added if which of the following are true:  (select more than one)

  - The customer recognizes the value

  - It reduces costs

  - It changes the product toward something the customer expects

  - It is done right the first time

  - it is important to business profitability

- Which elements are required to calculate Sigma?

  - Number of defects and standard deviation.

  - Process performance and customer requirements.

  - Cycle time and quality.

  - Number of defects, number of employees, number of customers.

- DPMO = 1M * D/(N*O).  A team has found that they can use DPMO to calculate Sigma.  They have collected the following production numbers for last week. 
The total number of units observed was 1251.
The number of defects was 34.  
The team has identified 6 opportunities per unit to not meet customer requirements.  A sigma Calculation table is below.  Calculate Sigma for this process.  DPMO =  1M * (D/(N*O)) 
   		DPMO = 1M * (34/(6*1251))

DPMO = 1M * (34/7506)

DPMO = 1M * (0.0045297)

DPMO = 4,529.7

### From SIGMA Calculation Table: between 2.6-2.7 Sigma

- Which of the following statements is NOT true about charting variation:

  - A mean and standard deviation will completely define a normal curve.

  - A histogram helps understand process performance.

  - A histogram is sufficient to chart variation.

  - Run charts display process performance over time.

- A team has collected data by counting defects.  Which type of control chart would they select?

  - Continuous

  - Discrete

  - EWMA

  - Individuals

  - X-bar

- Control chart limits for an individuals control chart are based roughly on:

  - 1 Standard Deviation from the mean.

  - 2 Standard Deviations from the mean.

  - 3 Standard Deviations from the mean.

  - Customer Specifications.

- A critical customer requirement must meet which if the following conditions

  - Important to the customer

  - Specifies “must have” or “must be” attributes

  - Can be measured

  - Establishes a target

  - All of the above.

- What are some of the reasons for selecting a sample? Any 2 or all 3.

  - The population is too large to measure all of it.

  - It would take too much time to collect all of the data.

  - Some of the data are inaccessible.

- What causes variation in the process?

  - Sales and marketing staff.

  - Customers.

  - Suppliers

  - Xs

  - Ys

- What is an operational definition?

  - A precise definition about how to operate a system.

  - A precise description of the criteria used for a measurement.

  - The methodology for collecting the data.

  - A precise way of defining a business process.

- Quality is defined by:

  - The CEO.

  - The chief engineer.

  - The plant manager.

  - The Six Sigma Team.

  - The customer.

  - The Black Belt.

  - The marketing director.

- A Black Belt in a production line has been given the following information to determine a sample to measure run times.  The standard deviation has been estimated at 2 days and they want to estimate run time within 6 hours.  What is the minimum sample size required for 95% confidence?  n= (1.96s/)2

n = (1.96 * 2) / .25)2

n = (3.92) / .25)2

n = (15.68)2

n = 245.86

The minimum sample size required is 246

- A control chart that is in control has:

  - Common cause variation

  - Special cause variation

  - A stable process

  - 10 data points

- Teams are important to Six Sigma because:

  - They include experts in key areas.

  - They bring different perspectives to the process.

  - They foster ownership of the problems and solutions.

  - They include key people though out the organization.

  - All of the above.

- The minimum number of points to construct a control chart should be:

  - 5-10 points

  - 20-30 points

  - 100-200 points

  - 200-400 points

  - more than 500

- The convention for calculating the yield for determining sigma is based on:

  - Final yield.

  - Rolled-throughput yield.

  - First-pass yield.

  - Combined yield.

  - Summarized yield.

- When working with team the term ‘Group Think’ means that:

  - Every one appears to agree even though they may not.

  - Every one agrees with the leader.

  - All members agree on the time schedule for the day.

  - Team members carefully consider every suggestion and decide as a group to approve or disapprove the suggestion.

- Chartering – List the six main elements of a charter.

  - Business Case

  - Opportunity Statement

  - Goal Statement

  - Project Scope

  - Project Plan

  - Team Selection

- List five components of a measurement plan.

  - Performance Measure

  - Operational Definition

  - Data source and Location

  - Sample Size

  - Who will collect the date?

  - When will the data be collected?

  - How will the data be collected?

  - Other data that should be collected

  - How will data be used?

  - How will data be displayed?

- What is a Critical Customer Requirement (CCR)?  The Specific, precise and measurable expectation which a customer has regarding a product or service.   The correct answer could also include the following: Important to the customer, specifies “must have” or “must be” attributes, can be measured, and establishes a target.

- What is a Critical to Quality (CTQ) measure? A process output measure that directly impacts a CCR.